TMAP Monitoring Handbook

Geomorphology

(version 15.12.2009, TMAG 09-3)

## 1. Introduction

The morphology of the Wadden Sea is constantly changing due to tidal current, other natural impact and anthropogenic activities.

Tidal flats are important feeding, roosting and/or moulting areas for birds at low tide and are also important habitats for seals, fish and shrimps. The high biological productivity of the tidal area sustains a high number of birds and fish and also offers advantages for fisheries.

Tidal flats are a habitat for several species which require a broad spectrum of sediment types. Due to changes in the sea level, the sediment composition of the tidal flats may change and would influence the benthic communities. Amongst others, human activities like fisheries may also alter the sediment composition of the tidal flats.

In the TMAP it is necessary to document dynamic and trendwise changes in the morphology of the Wadden Sea in order to obtain sound basic information in all three countries which can be used to assess whether these changes may result from anthropogenic activities.

## 2. Objectives

Trilateral policy and management aims “to achieve, as far as possible, a natural and sustainable ecosystem in which natural processes proceed in an undisturbed way” (Guiding Principle).

With respect to the “Tidal Area the following Target applies (Wadden Sea Plan):

- a natural dynamic situation in the Tidal Area,

- an increased area of geomorphologically and biologically undisturbed tidal flats and subtidal areas.

Monitoring of geomorphologic characteristics of the Wadden Sea is carried out to assess possible changes due to changes in climate (e.g. sea level rise, increase of storms) and the effect of these changes on the habitats, species and communities.

## 3. Monitoring requirements

Wadden Sea Plan

Targets on “Tidal Area”

- A natural dynamic situation in the tidal area.

- An increased area of geomorphologically and biologically undisturbed tidal flats and subtidal areas.

- A natural size, distribution and development of natural mussel beds, Sabellaria reefs and Zostera fields.

Habitats Directive (HD)

Monitoring of geomorphological features for habitat types in the Tidal Area:

Favorable conservation status of habitat types:

- Sandbanks (1110)

- Estuaries (1130)

- Mudflats (1140)

- Shallow inlets and bays (1160)

- Reefs (1170)

Water Framework Directive (WFD)

Article 8

Annex V, Chapter 1.2.3 and 1.2.4: Morphological conditions

OSPAR

No specific requirements for geomorphological monitoring.

## 4. Definitions

Tidal flats

Tidal flats are defined as mudflats and sandflats not covered by seawater at low tide (EU Habitats Directive) between Mean High Tide Level (MHTL) and Mean Low Tide Level (MLTL);

Sublitoral

Sediment types

| Sediment type | % clay (<2µm) | % silt (2-63 µm) |
| --- | --- | --- |
| Sand | < 5 | < 10 |
| Mixed | 5 - 8 | 10 - 50 |
| Mud | > 8 | > 50 |

(after Dijkema, 1987)

## 5. TMAP Monitoring Strategy

Monitoring of geomorphologic characteristics in the TMAP is carried out to assess possible changes due to changes in climate and the effect of these changes on habitats, species and communities. Information on area of tidal flats and sediment type distribution can be obtained from remote sensing surveys and in combination with existing field sampling (such as benthos monitoring).

Table 1: Monitoring of geomorphology.

| Parameters |  | Location | Frequency | WFD | BHD | OSPAR | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Area of tidal flats | Size and location (km²) | Entire area | 6 years | X | X | - |  |
| Sediment type distribution | Location and area of sediment types (sand, mixed, mud) | Entire area and/or selected sites | 6 years | X | X | - |  |

## 6. Methods

Ground truth and/or remote sensing methods or other appropriate comparable method should be applied (see also HARBASINS, 2006).

Several methods are used in the countries to monitor extension of tidal flats and sediment types for various purposes. The derived data can be regarded as valuable information, if it is comparable for the Wadden Sea area.

Remote sensing methods (optical satellite sensors) have been applied on project basis (OFEW, 2006 & 2007) and are still under development (DeMarine, 2009) but has not yet made operational for regular monitoring.

## 7. Parameters

- location and area (km²)

- extent of sediment types (km²) per sub-area or tidal basin: three types: mud, mixed (muddy sand) and sand (after Dijkema, 1989)

## 8. Frequency and time

- every 6 years

## 9. Monitoring locations

- all tidal flats in the Wadden Sea

- entire area or selected sites for sediment type distribution

## 10. Assessment

The assessment will be tuned with the WFD assessment procedure with is currently under development in the Wadden Sea countries (HARBASINS, 2007).

Table 2: Assessment of geomorphology in the Wadden Sea (preliminary).

| Parameter |  | Assessment / Objective |
| --- | --- | --- |
| Area of tidal flats | km² | Stable or natural fluctuation |
| Sediment type distribution | km² of types per sub-area (tidal basin, region) | Stable or natural fluctuation |

## 11. Reporting

In the framework of the preparation of trilateral reports or a QSR, the requested information should be made available by the responsible authorities: tables with information about changes in occurrence, distribution and size of sediment types of tidal flat areas (including GIS data).

The information should be reported at least every six years to the national TMAP data unit (according to the national regulations) to be available for trilateral assessments (see TMAP Data Handling Manual).

## 12. Quality Assurance

Appropriate Quality Assurance (QA) activities should be in place and reported together with information about applied methods.

## 13. Monitoring Authorities

Denmark

- Kystinspektoratet Lemvig

- Danmarks Miljøundersøgelser (DMU, NERI)

- Miljøministeriet, Miljøcenter Ribe

Germany

- Bundesamt für Seeschifffahrt und Hydrographie, Hamburg (BSH)

- Landesamt für Landwirtschaft, Umwelt und ländliche Räume (LLUR)

- Landesbetrieb für Küstenschutz, Nationalpark und Meeresschutz (LKN)

- Niedersächsischer Landesbetrieb für Wasserwirtschaft, Küsten- und Naturschutz (NLWKN)

The Netherlands

- Rijkswaterstaat Waterdienst, Lelystad

- Rijkswaterstaat Noord, Leeuwarden

## 14. Literature

DeMarine Umwelt, 2009. Jahresbericht Teilprojekt 4, Integration optischer und SAR Erdbeobach-tungsdaten für das Wattenmeermonitoring.

http://www.demarine-umwelt.de/Dokumente/DeMU_Jahresbericht_2009.pdf

HARBASINS, 2007. Synergies in the WFD implementation in the Wadden Sea. Report from a trilateral workshop, Hamburg, 24 – 25 October 2007.

HARBASINS, 2006. Workshop on Remote Sensing of Intertidal Flats. Presentations and Practical Exercises. Brockmann Consult and Common Waddden Sea Secretariat. Proceeding of a workshop at the Geesthachter Innovations- und Technologiezentrum (GITZ), 11 - 12 September 2006.

Stelzer, K., Brockmann, C. & J. Geißler, 2007. Abschlussbericht OFEW – Operationalisierung von Fernerkundungsmethoden für das Wattenmeermonitoring. Version 1, Überarbeitungsdatum 10.09.2007.